SOFTWARE REQUIREMENTS SPECIFICATION & SYSTEM DESIGN DOCUMENT

Edge AI Based Smart EV Charging Station Optimizer

Using ESP32, MQTT and ThingsBoard

Final Year Project — IoT, Edge AI, Smart Energy Management

| Document Type | Version | Date |
| --- | --- | --- |
| SRS & SDD | 1.0 | June 2026 |

# 1. Introduction

## 1.1 Purpose of the Document

This document specifies the functional and non-functional requirements, system architecture, and detailed design for the Edge AI Based Smart EV Charging Station Optimizer. It serves as a reference for development, simulation, evaluation, and final project report writing. The document covers both the what (requirements) and the how (design and implementation approach) using ESP32 (or Arduino Uno with a WiFi module), MQTT, and ThingsBoard.

## 1.2 Project Background and Motivation

With the rapid growth of electric vehicle (EV) adoption, charging infrastructure is under increasing strain. Charging stations, especially in commercial and residential complexes, often operate without intelligence — chargers are allocated on a first-come-first-served basis, electricity is drawn without regard to tariff structure, and station operators have no visibility into usage patterns. This leads to inefficiency for both EV owners and station operators.

By embedding a lightweight AI model at the edge (on the ESP32 controlling each charging point) and connecting it to a cloud dashboard (ThingsBoard) via MQTT, the station can make local, low-latency decisions (e.g., which charger to activate, when to throttle power) while still giving operators a centralized view and historical analytics.

## 1.3 Problem Statement

- Long waiting times — EV owners cannot tell in advance which charger will be free or how long they must wait.

- Uneven charger utilization — some charging points are overused while others remain idle, due to lack of coordination.

- Peak-hour congestion — multiple vehicles arriving at the same time overload the available chargers and the local power supply.

- Increased electricity costs during peak demand — charging during high-tariff periods increases operating costs for the station owner.

- Lack of predictive analytics — station operators have no forecast of upcoming demand, so they cannot plan power allocation, staffing, or maintenance.

## 1.4 Proposed Solution Overview

The proposed system consists of multiple ESP32-based charging point controllers (one per charging bay), each instrumented with voltage, current, and (simulated) battery State-of-Charge (SoC) sensing. Each ESP32:

- Continuously measures charging parameters (voltage, current, power, energy consumed, temperature).

- Runs a lightweight Edge AI model that predicts short-term charging demand (e.g., probability of a new EV arriving in the next 15/30/60 minutes, and expected session duration) using historical and time-context features.

- Uses this prediction, combined with the current load of all bays (received via MQTT), to make a local allocation/throttling decision — e.g., enabling/disabling a relay, suggesting an available bay, or shifting load to off-peak hours.

- Publishes telemetry and prediction results to ThingsBoard over MQTT, where dashboards visualize charger status, predicted demand, energy cost, and alerts in real time.

- Receives remote commands/configuration (e.g., manual override, tariff schedule updates) from ThingsBoard via MQTT RPC.

The result is a station that balances load automatically, reduces peak-hour cost, gives EV owners better wait-time estimates, and gives operators a predictive, centralized dashboard.

## 1.5 Objectives

- To design and simulate a multi-charger EV station controller using ESP32 (or Arduino Uno + ESP8266) in a circuit simulator (Wokwi).

- To implement sensor-based monitoring of voltage, current, power, and energy for each simulated charging bay.

- To develop a lightweight Edge AI model for short-term demand prediction that can run on a microcontroller.

- To implement an optimization/allocation algorithm that assigns chargers and manages load based on predictions.

- To integrate the system with ThingsBoard via MQTT for real-time monitoring, historical analytics, alerting, and remote control.

- To evaluate the system's effectiveness in reducing peak load and improving charger utilization through simulation.

## 1.6 Scope of the Project

### In Scope

- Simulation of 2–4 EV charging bays using ESP32 in Wokwi, with simulated sensors (potentiometers/virtual sensors representing current and voltage).

- An Edge AI model (lightweight regression / decision tree, deployable as C code or TensorFlow Lite Micro) trained offline on a synthetic/historical dataset and embedded in the ESP32 firmware.

- MQTT-based communication between ESP32 nodes and ThingsBoard (cloud instance or local ThingsBoard CE via Docker).

- ThingsBoard dashboards for live monitoring, demand prediction charts, alarms, and remote relay control.

- A rule-chain on ThingsBoard for alerting (overload, high predicted demand, abnormal readings).

### Out of Scope

- Real high-voltage EV charging hardware (CCS/CHAdeMO connectors, actual battery management systems).

- Payment gateway / billing integration.

- Mobile application development (dashboard access is via ThingsBoard's web/mobile portal only).

- Large-scale, multi-station deployment (the project demonstrates the concept on a single station with multiple bays).

## 1.7 Assumptions and Constraints

- The project will be demonstrated primarily through simulation (Wokwi for ESP32 hardware, ThingsBoard cloud/demo or local CE instance for the dashboard).

- “Battery SoC” and “EV arrival” events are simulated using potentiometers, push buttons, or randomized/synthetic data generators, since real EVs are not available.

- Internet connectivity is required for MQTT communication to ThingsBoard (Wokwi supports outbound internet via its built-in WiFi simulation).

- The Edge AI model must be small enough (a few KB) to fit within ESP32's flash/RAM alongside the WiFi/MQTT stack.

- If Arduino Uno is used instead of ESP32, an ESP8266 WiFi module (or ESP-01) is required for MQTT connectivity, and the Edge AI model must be reduced to a very simple rule-based/linear model due to the Uno's limited memory (2 KB SRAM, 32 KB flash).

## 1.8 Definitions, Acronyms and Abbreviations

| Term | Description |
| --- | --- |
| EV | Electric Vehicle |
| Edge AI | Artificial Intelligence inference performed locally on the device (microcontroller) rather than in the cloud |
| MQTT | Message Queuing Telemetry Transport — lightweight publish/subscribe messaging protocol |
| ThingsBoard | Open-source IoT platform for data collection, visualization, processing, and device management |
| SoC | State of Charge (battery charge percentage) |
| QoS | Quality of Service (MQTT message delivery guarantee level) |
| RPC | Remote Procedure Call (used by ThingsBoard to send commands to devices) |
| TFLite Micro | TensorFlow Lite for Microcontrollers |
| DFD | Data Flow Diagram |
| OTA | Over-The-Air (firmware update) |

# 2. Overall System Description

## 2.1 System Perspective

Charging bay ESP32 nodes connect over MQTT to ThingsBoard's built-in broker. ThingsBoard hosts the device registry, dashboards, rule chains, alarms/notifications, telemetry and attribute storage, and issues RPC commands back down to devices.

## 2.2 High-Level System Functions

- Sensing — measure voltage, current, power, energy consumed, and ambient temperature at each charging bay.

- Edge inference — predict short-term demand (likelihood of a new EV arriving / how long the current EV will charge) using an on-device AI model.

- Local optimization — decide whether to enable/throttle/disable a charger based on the prediction, current load, and a simple priority/cost rule.

- Cloud telemetry — publish sensor readings, predictions, and decisions to ThingsBoard over MQTT at a configurable interval.

- Visualization — display live and historical data, predicted demand curves, charger status, and alerts on ThingsBoard dashboards.

- Remote control — allow the operator to override charger states or update configuration (e.g., tariff schedule, thresholds) from ThingsBoard via RPC.

- Alerting — raise alarms in ThingsBoard for overload conditions, abnormal sensor values, or high predicted demand.

## 2.3 User Classes and Stakeholders

| User Class | Description | Interaction |
| --- | --- | --- |
| EV Owner | Person charging their vehicle | Indirect — benefits from reduced wait time and balanced load |
| Station Operator / Admin | Manages the charging station | Uses ThingsBoard dashboard for monitoring, analytics, and remote control |
| Project Evaluator | Examines the academic project | Reviews the dashboard, simulation, and this document |
| System (Edge AI Optimizer) | Autonomous decision-making agent | Runs on each ESP32, makes allocation/throttling decisions |

## 2.4 Operating Environment

- Hardware (simulated): ESP32 Dev Module (or Arduino Uno + ESP8266), ACS712 current sensor, voltage divider/ZMPT101B voltage sensor, relay module, DHT11/DS18B20 temperature sensor, OLED/LCD (optional), push buttons/potentiometers for simulating EV arrival and SoC.

- Simulation Platform: Wokwi (wokwi.com) for ESP32/Arduino circuit and firmware simulation with internet access.

- Firmware: Arduino C/C++ using Arduino IDE or PlatformIO.

- Cloud Platform: ThingsBoard Community Edition — either the public demo or a self-hosted instance via Docker.

- Communication Protocol: MQTT (v3.1.1), using ThingsBoard's device MQTT API.

- AI Model Development: Python (scikit-learn / pandas) for offline training; model exported as C arrays (via m2cgen or micromlgen) or as a TensorFlow Lite Micro .tflite model for embedding in firmware.

# 3. Functional Requirements

| ID | Requirement | Description |
| --- | --- | --- |
| FR-1 | Sensor Data Acquisition | The system shall read voltage, current, and temperature values from each charging bay at a configurable sampling interval (default: every 5 seconds). |
| FR-2 | Power & Energy Calculation | The system shall compute instantaneous power (W) and cumulative energy (Wh/kWh) for each charging bay from voltage and current readings. |
| FR-3 | Simulated EV Arrival/Departure Detection | The system shall detect simulated EV plug-in/plug-out events (via push button or threshold change in current) and update bay status (Free / Charging / Fault). |
| FR-4 | Edge AI Demand Prediction | Each ESP32 shall run an on-device AI model to predict (a) probability of a new arrival in the next time window, and (b) estimated remaining charging time for the current session, using time-of-day, day-of-week, and historical usage features. |
| FR-5 | Charger Allocation Optimization | The system shall use the demand predictions and current bay status (shared between nodes via MQTT or aggregated in ThingsBoard) to recommend/activate the most suitable available charger for an incoming EV, balancing load across bays. |
| FR-6 | Peak-Load Management | The system shall throttle or stagger charging (via relay duty-cycling) when the combined predicted/actual load exceeds a configurable threshold, prioritizing sessions based on configurable rules. |
| FR-7 | MQTT Telemetry Publishing | Each ESP32 shall publish telemetry (voltage, current, power, energy, temperature, bay status, predicted demand, allocation decision) to ThingsBoard via MQTT in JSON format at a configurable interval. |
| FR-8 | MQTT Attribute & RPC Handling | Each ESP32 shall subscribe to shared attribute updates and RPC requests from ThingsBoard to receive configuration changes and manual override commands. |
| FR-9 | Dashboard Visualization | ThingsBoard shall display, per charging bay: live status, voltage/current/power graphs, energy consumed, predicted demand curve, and allocation history. |
| FR-10 | Alarms & Notifications | ThingsBoard rule chains shall raise alarms for: overcurrent/overvoltage, sensor disconnection/fault, predicted demand exceeding capacity, and prolonged “stuck” sessions. |
| FR-11 | Historical Data Logging | ThingsBoard shall retain time-series telemetry for trend analysis (utilization over hours/days, peak vs. off-peak comparison). |
| FR-12 | Local Display (optional) | Each ESP32 may show current status (Free/Charging/Predicted Wait) on a local OLED/LCD for demonstration purposes. |

# 4. Non-Functional Requirements

| Category | Requirement |
| --- | --- |
| Performance | Sensor sampling and MQTT publish cycle shall complete within the configured interval (≤5 s) without blocking; Edge AI inference shall complete in under 200 ms on ESP32. |
| Scalability | The architecture shall support adding additional charging bay nodes (ESP32 units) without redesigning the cloud/dashboard layer — each node is provisioned as a separate ThingsBoard device. |
| Reliability | The system shall automatically reconnect to WiFi and the MQTT broker after connectivity loss, and shall buffer/discard telemetry gracefully without crashing. |
| Security | MQTT communication with ThingsBoard shall use per-device access tokens (or X.509 certificates for production); RPC commands shall be validated before execution. |
| Usability | Dashboards shall present information using clear widgets (gauges, time-series charts, status indicators) understandable to a non-technical station operator. |
| Maintainability | Firmware shall be modular (separate modules for sensing, AI inference, MQTT communication, and control logic) to allow independent updates. |
| Portability | The firmware shall be portable between ESP32 and Arduino Uno + ESP8266 with minimal changes (abstracted WiFi/MQTT layer). |
| Energy Efficiency | The optimization logic shall aim to reduce total energy drawn during defined peak-tariff hours compared to an unmanaged baseline. |

# 5. Hardware Requirements

## 5.1 Primary Option — ESP32-Based Node (Recommended)

| Component | Purpose | Simulation Equivalent (Wokwi) |
| --- | --- | --- |
| ESP32 Dev Module (WROOM-32) | Main controller — WiFi, MQTT, Edge AI inference | wokwi-esp32-devkit-v1 |
| ACS712 Current Sensor | Measures charging current | Potentiometer mapped to current range |
| Voltage Sensor (divider / ZMPT101B) | Measures supply/charging voltage | Potentiometer mapped to voltage range |
| Relay Module (5V, 1-ch per bay) | Switches charging current ON/OFF | wokwi-relay-module |
| DHT22 / DS18B20 | Ambient/charger temperature monitoring | wokwi-dht22 |
| Push Buttons (×2 per bay) | Simulate EV plug-in / plug-out events | wokwi-pushbutton |
| 16x2 LCD or SSD1306 OLED (optional) | Local status display | wokwi-lcd1602 / wokwi-ssd1306 |
| LEDs (Green/Red/Yellow) | Visual status indicator per bay | wokwi-led |

## 5.2 Alternative Option — Arduino Uno + ESP8266 Node

| Component | Purpose |
| --- | --- |
| Arduino Uno | Sensor reading, relay control, and rule-based (non-AI or minimal AI) logic |
| ESP8266 (ESP-01 / NodeMCU) | Provides MQTT connectivity to ThingsBoard |
| ACS712, voltage divider, relay, push buttons | Same as the primary option |

Note: Because Arduino Uno has only 2 KB SRAM and 32 KB flash, the Edge AI model must be reduced to a simple linear regression or small decision tree. The ESP32 option is strongly recommended, as it has enough RAM/flash to host both the MQTT stack and a compact ML model.

# 6. Software Requirements

| Tool / Library | Purpose |
| --- | --- |
| Arduino IDE 2.x or PlatformIO | Firmware development for ESP32/Arduino |
| Wokwi Simulator (web-based) | Circuit design and firmware simulation with internet access |
| WiFi.h (ESP32 core) | WiFi connectivity |
| PubSubClient | MQTT client library |
| ArduinoJson | JSON serialization/deserialization for MQTT payloads |
| Adafruit_SSD1306 / LiquidCrystal (optional) | Local display drivers |
| ThingsBoard CE (Cloud or self-hosted via Docker) | Device management, telemetry storage, dashboards, rule chains, alarms |
| Python 3.x (pandas, scikit-learn, numpy) | Offline training of the demand-prediction Edge AI model |
| micromlgen / m2cgen | Convert trained scikit-learn model to embeddable C/C++ code |
| TensorFlow Lite Micro (optional) | Alternative deployment route for neural-network-based models |

# 7. System Architecture

## 7.1 Three-Layer Architecture

The system follows a classic Edge–Communication–Cloud architecture:

### Layer 1 — Edge Layer (ESP32 Nodes)

Each charging bay has a dedicated ESP32 that performs sensing, runs the Edge AI inference, executes the local allocation/throttling logic, and controls the relay. This layer operates with low latency and can continue basic operation even if cloud connectivity is temporarily lost.

### Layer 2 — Communication Layer (MQTT)

All ESP32 nodes connect to ThingsBoard's MQTT broker using individual device access tokens. Telemetry is published on the standard ThingsBoard topic v1/devices/me/telemetry, attributes on v1/devices/me/attributes, and RPC commands are received on v1/devices/me/rpc/request/+ and responded to on v1/devices/me/rpc/response/+.

### Layer 3 — Cloud Layer (ThingsBoard)

ThingsBoard stores time-series telemetry, runs rule chains for alarm generation and cross-node aggregation (e.g., total station load), and serves real-time dashboards to the operator. Shared attributes (tariff schedule, thresholds) are pushed down to devices when updated.

## 7.2 Edge AI Placement

The Edge AI demand-prediction model runs locally on each ESP32. This design choice reduces dependence on constant connectivity for core decision-making, reduces cloud compute cost, and demonstrates the “Edge AI” concept central to the project — inference happens at the point of data generation. ThingsBoard remains responsible for cross-node aggregation, visualization, historical analytics, and operator alerts.

## 7.3 MQTT Topic Design

| Topic | Direction | Purpose |
| --- | --- | --- |
| v1/devices/me/telemetry | ESP32 → ThingsBoard | Publish sensor readings, predictions, and allocation status |
| v1/devices/me/attributes | ESP32 → ThingsBoard | Publish client-side attributes (firmware version, IP address) |
| v1/devices/me/attributes (subscribe) | ThingsBoard → ESP32 | Receive shared attribute updates (thresholds, tariff schedule) |
| v1/devices/me/rpc/request/+ | ThingsBoard → ESP32 | Receive RPC commands (e.g., setRelayState, setMode) |
| v1/devices/me/rpc/response/+ | ESP32 → ThingsBoard | Send RPC acknowledgement/result |

# 8. Detailed Design

## 8.1 Per-Bay Hardware/Wiring Design (Wokwi)

- Voltage sensing: a potentiometer (simulating a voltage divider) on ESP32 ADC GPIO34, scaled in firmware to represent 0–250 V.

- Current sensing: a second potentiometer (simulating ACS712 output) on GPIO35, scaled to represent 0–32 A.

- Temperature sensing: DHT22 connected to GPIO15.

- Relay: connected to GPIO26, controlling the simulated charging circuit.

- EV plug-in/out buttons: GPIO32 (plug-in) and GPIO33 (plug-out).

- Status LEDs: Green (GPIO18, Free), Yellow (GPIO19, Charging), Red (GPIO21, Fault/Overload).

- OLED (optional): I2C SDA=GPIO21*, SCL=GPIO22 — if OLED I2C uses GPIO21, relocate the Red LED to GPIO23.

## 8.2 Data Flow Diagram (Level 1)

Voltage/Current/Temp Sensors
        |
        v
Sensor Read & Calibration Module
        |
        v
Power/Energy Calculator
        |
        v
Feature Builder (time, day, history, V/I/Temp)
        |
        v
Edge AI Inference Engine (Demand & Duration Predictor)
        |
        v
Local Optimization / Allocation Logic
  - Compare with neighbor bay loads
  - Apply threshold & priority rules
  - Decide relay state / throttle level
        |                          |
        v                          v
Relay / LED Control      MQTT Telemetry Publisher
                                    |
                                    v
                          ThingsBoard Cloud
                       (Dashboard, Alarms, RPC)

## 8.3 MQTT Payload (Telemetry) Schema

{
  "bayId": "BAY_01",
  "voltage": 228.4,
  "current": 14.2,
  "power": 3243.3,
  "energyWh": 1520.6,
  "temperature": 34.5,
  "bayStatus": "CHARGING",
  "predictedArrivalProb": 0.62,
  "predictedDurationMin": 42,
  "loadDecision": "ALLOW",
  "throttleLevel": 100
}

| Field | Type | Description |
| --- | --- | --- |
| bayId | string | Identifier for the charging bay/device |
| voltage | float | Measured voltage (V) |
| current | float | Measured current (A) |
| power | float | Instantaneous power, computed as V × I (W) |
| energyWh | float | Cumulative energy consumed in current session (Wh) |
| temperature | float | Ambient/charger temperature (°C) |
| bayStatus | string | One of FREE, CHARGING, FAULT |
| predictedArrivalProb | float | Edge AI output: probability of a new EV arriving within the next prediction window (0–1) |
| predictedDurationMin | int | Edge AI output: estimated remaining charging time (minutes) |
| loadDecision | string | Result of local optimization: ALLOW, THROTTLE, DEFER |
| throttleLevel | int | Percentage of full charging current permitted (0–100) |

## 8.4 Shared Attributes (Cloud → Device Configuration)

| Attribute | Type | Description |
| --- | --- | --- |
| maxStationLoadW | float | Maximum total power the station can draw (used for peak-load checks) |
| peakTariffStart / peakTariffEnd | int (hour) | Defines peak-tariff window for cost-aware scheduling |
| overloadCurrentA | float | Current threshold above which an overload alarm is raised |
| predictionThreshold | float | Probability above which the system pre-emptively reserves capacity |

## 8.5 RPC Commands

| Method | Direction | Params | Description |
| --- | --- | --- | --- |
| setRelayState | ThingsBoard → ESP32 | { "state": true/false } | Manual override to force charger ON/OFF |
| setThrottle | ThingsBoard → ESP32 | { "level": 0-100 } | Manually set throttle percentage |
| getStatus | ThingsBoard → ESP32 | — | Request immediate status report |

## 8.6 Edge AI Model Design

### 8.6.1 Objective

Predict, for each charging bay and at each evaluation cycle (e.g., every 15 minutes): (1) arrival probability — likelihood that a new EV will plug in within the next 15/30/60-minute window; (2) session duration — estimated remaining charging time for an active session, based on current power draw trend and historical session lengths.

### 8.6.2 Input Features

| Feature | Description |
| --- | --- |
| hourOfDay (0–23) | Captures daily usage patterns (e.g., morning/evening peaks) |
| dayOfWeek (0–6) | Captures weekday vs. weekend patterns |
| currentBayStatus | Whether the bay is currently free or occupied |
| recentAvgCurrent | Rolling average current over the last N samples (trend indicator) |
| sessionElapsedMin | Minutes since the current session started (if occupied) |
| neighborBaysOccupied | Count of other bays currently occupied (station-wide congestion indicator) |
| historicalArrivalRate | Pre-computed average arrivals for this hour/day from training data |

### 8.6.3 Model Selection

Arrival probability: a small Logistic Regression or shallow Decision Tree (depth ≤ 5) trained on a synthetic/historical dataset of (time features → arrival within window) records. Session duration: a small Linear Regression or Decision Tree Regressor trained on (session features → actual duration) records. These models are converted to plain C code using micromlgen (for trees) or m2cgen (for linear/logistic models), producing a header file (model.h) included directly in the Arduino sketch — no TensorFlow runtime required, keeping the footprint under a few KB.

### 8.6.4 Training Data Source

Since real station data is unavailable, a synthetic dataset is generated reflecting realistic patterns: higher arrival probability during 8–10 AM and 6–9 PM (commute hours); lower arrival probability late at night (12–5 AM); session durations modeled as a function of arrival SoC (random 20–80%) and a fixed/variable charging rate. A Python script generates this dataset (5,000–10,000 rows), trains the two models, evaluates accuracy/R², and exports them via micromlgen/m2cgen.

### 8.6.5 On-Device Inference Flow

- Every prediction cycle, the ESP32 builds the feature vector from current sensor readings, the internal clock (via NTP-synced time), and rolling statistics stored in RAM.

- The feature vector is passed to the generated predict() functions (arrival model and duration model) from model.h.

- Outputs (predictedArrivalProb, predictedDurationMin) are stored and included in the next MQTT telemetry publish.

## 8.7 Charger Allocation / Optimization Algorithm (Pseudocode)

EVERY optimization cycle (e.g., every 30 seconds):
 
  read localBayStatus, localPower, predictedArrivalProb, predictedDurationMin
  read neighborBayStatuses (via MQTT shared telemetry / ThingsBoard attribute)
  read maxStationLoadW, overloadCurrentA, peakTariffStart/End (shared attributes)
 
  totalStationPower = sum(localPower, neighborPowers)
 
  IF localCurrent > overloadCurrentA:
        loadDecision = "THROTTLE"; throttleLevel = 50; raise local alarm flag
 
  ELSE IF totalStationPower > maxStationLoadW:
        IF predictedDurationMin > median(neighborDurations):
              loadDecision = "THROTTLE"; throttleLevel = 70
        ELSE:
              loadDecision = "ALLOW"; throttleLevel = 100
 
  ELSE IF currentHour in [peakTariffStart, peakTariffEnd]
         AND localBayStatus == FREE
         AND predictedArrivalProb < predictionThreshold:
        loadDecision = "DEFER"; throttleLevel = 0
 
  ELSE:
        loadDecision = "ALLOW"; throttleLevel = 100
 
  apply throttleLevel to relay / PWM control
  publish telemetry including loadDecision, throttleLevel, predictions

This logic runs independently on each ESP32 but uses shared station-wide context (via ThingsBoard shared attributes / cross-subscription) to coordinate decisions — a simple form of distributed edge optimization.

## 8.8 ThingsBoard Dashboard Design

| Widget | Data Source | Purpose |
| --- | --- | --- |
| Status Card (per bay) | bayStatus | Shows FREE / CHARGING / FAULT with color coding |
| Gauge — Power (W) | power | Real-time power draw per bay |
| Time-Series — Voltage & Current | voltage, current | Trend monitoring |
| Time-Series — Total Station Load | Aggregated power (rule chain) | Peak-load visualization |
| Bar/Line — Predicted Arrival Probability | predictedArrivalProb | Demand forecast visualization |
| Table — Allocation Decisions | loadDecision, throttleLevel | Audit of optimization actions |
| Alarm Widget | ThingsBoard Alarms | Displays active alarms (overload, fault, high demand) |
| Control Widget — Manual Relay Override | RPC setRelayState | Operator override per bay |
| Energy Consumption Chart | energyWh (delta over time) | Daily/weekly energy usage per bay |

## 8.9 ThingsBoard Rule Chain Design

| Rule Node | Trigger | Action |
| --- | --- | --- |
| Overcurrent Check | current > overloadCurrentA | Create “Critical” alarm: “Overload on {bayId}” |
| Sensor Fault Check | No telemetry received for > 60s | Create “Major” alarm: “Device offline: {bayId}” |
| High Demand Forecast | predictedArrivalProb > predictionThreshold AND all bays CHARGING | Create “Warning” alarm: “High demand expected, no free bays” |
| Aggregation Node | On telemetry from any bay | Compute totalStationPower; save as asset-level telemetry |
| Tariff Update | Scheduled (cron) | Push updated peakTariffStart/End shared attributes to all devices |

# 9. Implementation Plan

## 9.1 Phase-Wise Plan

| Phase | Tasks | Output |
| --- | --- | --- |
| Phase 1 — Setup | Set up Wokwi project, wire ESP32 with sensors/relays/LEDs for one bay; set up ThingsBoard account/instance and create a device | Working circuit + ThingsBoard device with access token |
| Phase 2 — Sensing & Telemetry | Implement sensor reading, power/energy calculation, and MQTT telemetry publishing | Live data visible on ThingsBoard dashboard |
| Phase 3 — Edge AI | Generate synthetic dataset, train models in Python, export with micromlgen/m2cgen, integrate model.h into firmware | On-device predictions appearing in telemetry |
| Phase 4 — Optimization Logic | Implement allocation/throttling algorithm, relay control, and shared-attribute handling | Relay responds to load/prediction conditions |
| Phase 5 — Multi-Bay Scaling | Duplicate the node design for 2–4 bays, implement cross-node load aggregation via ThingsBoard rule chain | Multi-bay dashboard with total station load |
| Phase 6 — Dashboards, Alarms & RPC | Build dashboard widgets, configure rule chains for alarms, implement RPC for manual override | Complete operator dashboard |
| Phase 7 — Testing & Evaluation | Run simulation scenarios (normal load, peak load, sensor fault), record results | Test report + performance evaluation |

## 9.2 Wokwi Simulation Setup (Summary Steps)

- Create a new Wokwi project, select ESP32 as the board.

- Add components: 2× potentiometers (voltage/current), DHT22, relay module, 2× push buttons, status LEDs, optional SSD1306 OLED.

- Wire components to ESP32 GPIOs as per Section 8.1.

- In Wokwi's diagram.json, ensure the ESP32's WiFi is connected to Wokwi-GUEST (Wokwi's simulated internet access), which allows the device to reach ThingsBoard's MQTT broker.

- Write and upload the Arduino sketch (Section 9.3) via the Wokwi code editor; run the simulation.

## 9.3 Sample ESP32 Firmware (Arduino Sketch) — Key Excerpts

The full sketch (WiFi/MQTT setup, sensor sampling, Edge AI inference call, optimization decision, and telemetry publish) follows the structure below. See the project firmware files for the complete, buildable source.

#include <WiFi.h>
#include <PubSubClient.h>
#include <ArduinoJson.h>
#include "model.h"   // predictArrival(), predictDuration()
 
const char* WIFI_SSID = "Wokwi-GUEST";
const char* TB_SERVER = "demo.thingsboard.io";
const int   TB_PORT   = 1883;
const char* TB_TOKEN  = "YOUR_DEVICE_ACCESS_TOKEN";
 
#define VOLTAGE_PIN 34
#define CURRENT_PIN 35
#define RELAY_PIN   26
#define BTN_PLUGIN  32
#define BTN_PLUGOUT 33
#define LED_GREEN   18
#define LED_YELLOW  19
#define LED_RED     23
 
float maxStationLoadW = 6000.0;
float overloadCurrentA = 16.0;
float predictionThreshold = 0.5;

void loop() {
  if (!client.connected()) reconnectMQTT();
  client.loop();
 
  if (millis() - lastSampleTime >= 5000) {
    lastSampleTime = millis();
 
    float voltage = map(analogRead(VOLTAGE_PIN), 0, 4095, 0, 250);
    float current = map(analogRead(CURRENT_PIN), 0, 4095, 0, 320) / 10.0;
    float power = voltage * current;
 
    // plug-in / plug-out, energy accumulation, feature build...
    float predictedArrivalProb = predictArrival(features);
    int   predictedDurationMin = (int)predictDuration(features);
 
    String loadDecision = "ALLOW";
    int throttleLevel = 100;
    if (current > overloadCurrentA) {
      loadDecision = "THROTTLE"; throttleLevel = 50;
    } else if (power > maxStationLoadW) {
      loadDecision = "THROTTLE"; throttleLevel = 70;
    } else if (bayStatus == "FREE" && predictedArrivalProb < predictionThreshold) {
      loadDecision = "DEFER"; throttleLevel = 0;
    }
 
    // ...serialize JSON and client.publish("v1/devices/me/telemetry", buffer);
  }
}

Note: model.h is generated offline (Section 9.5) and provides predictArrival() and predictDuration(). For a first working version without the AI model, these two functions can be stubbed (e.g., return a fixed value or a simple time-based lookup table) so the MQTT/ThingsBoard pipeline can be validated independently before integrating the trained model.

## 9.4 ThingsBoard Setup Steps

- Create a free account at thingsboard.cloud (or deploy ThingsBoard CE locally via Docker).

- Under Devices, create one device per charging bay (e.g., BAY_01, BAY_02), and copy each device's access token.

- Paste the access token into the corresponding ESP32 sketch (TB_TOKEN).

- Under Dashboards, create a new dashboard and add widgets bound to each device's telemetry keys (Section 8.8).

- Under Rule Chains, edit the root rule chain and add the nodes described in Section 8.9 for alarms and aggregation.

- Define Shared Attributes (maxStationLoadW, overloadCurrentA, predictionThreshold, peakTariffStart, peakTariffEnd) for each device.

## 9.5 Edge AI Model Training & Export (Python, Offline)

import pandas as pd
from sklearn.linear_model import LogisticRegression
from sklearn.tree import DecisionTreeRegressor
from micromlgen import port
import m2cgen as m2c
 
df = pd.read_csv("synthetic_ev_data.csv")
 
X_arrival = df[["hourOfDay", "dayOfWeek", "bayOccupied", "recentAvgCurrent", "sessionElapsedMin"]]
y_arrival = df["arrivedWithinWindow"]
 
X_duration = df[df.bayOccupied == 1][["hourOfDay", "dayOfWeek", "bayOccupied", "recentAvgCurrent", "sessionElapsedMin"]]
y_duration = df[df.bayOccupied == 1]["actualDurationMin"]
 
arrival_model = LogisticRegression(max_iter=500).fit(X_arrival, y_arrival)
duration_model = DecisionTreeRegressor(max_depth=5).fit(X_duration, y_duration)
 
arrival_c_code = m2c.export_to_c(arrival_model)
duration_c_code = port(duration_model)
 
with open("model.h", "w") as f:
    f.write(arrival_c_code.replace("score", "predictArrival"))
    f.write(duration_c_code.replace("score", "predictDuration"))

The resulting model.h is copied into the Arduino sketch's project folder so it can be #included directly, as shown in Section 9.3.

# 10. Testing Plan

| Test ID | Scenario | Input / Action | Expected Result |
| --- | --- | --- | --- |
| T-01 | Telemetry publishing | Power on ESP32 with valid WiFi/MQTT config | Telemetry appears on ThingsBoard within 10 s; values match simulated sensor inputs |
| T-02 | EV plug-in detection | Press “plug-in” button | bayStatus changes FREE → CHARGING; energy counter starts incrementing |
| T-03 | EV plug-out detection | Press “plug-out” button | bayStatus reverts to FREE; energy counter stops |
| T-04 | Overcurrent handling | Set current pot above overloadCurrentA | loadDecision = THROTTLE, red LED ON, alarm raised on ThingsBoard |
| T-05 | Peak-load throttling (multi-bay) | Low maxStationLoadW; 2+ bays at high power | Bay(s) with higher predictedDurationMin switch to THROTTLE |
| T-06 | Demand prediction sanity | Simulate different hourOfDay values | predictedArrivalProb higher during peak hours than off-peak |
| T-07 | Remote relay override (RPC) | Send setRelayState RPC from ThingsBoard | Relay state changes immediately; bayStatus/LEDs reflect new state |
| T-08 | Shared attribute update | Update overloadCurrentA from ThingsBoard | New threshold takes effect on next evaluation cycle, no reflash |
| T-09 | Connectivity loss recovery | Disconnect WiFi temporarily | ESP32 reconnects automatically; telemetry resumes |
| T-10 | Sensor fault alarm | Stop telemetry from one device for >60s | ThingsBoard raises a “Device offline” alarm for that bay |

# 11. Project Timeline (Suggested, 10 Weeks)

| Week | Activity |
| --- | --- |
| 1 | Literature review, finalize requirements, set up Wokwi & ThingsBoard accounts |
| 2 | Design circuit for single bay; wire sensors, relay, LEDs in Wokwi |
| 3 | Implement sensor reading, power/energy calculation; basic MQTT telemetry to ThingsBoard |
| 4 | Build ThingsBoard dashboard for single bay; configure device attributes |
| 5 | Generate synthetic dataset; train and export Edge AI models (Python) |
| 6 | Integrate model.h into firmware; validate predictions in telemetry |
| 7 | Implement local optimization/allocation algorithm and relay throttling |
| 8 | Replicate design for 2–4 bays; implement cross-bay aggregation via rule chain |
| 9 | Configure alarms, RPC override controls, finalize dashboards |
| 10 | Run test scenarios (Section 10), collect results, prepare final report and presentation |

# 12. Expected Outcomes

- A working multi-bay EV charging simulation where each bay independently senses, predicts demand, and makes load/allocation decisions.

- A live ThingsBoard dashboard showing real-time and historical data, predicted demand trends, and active alarms.

- Demonstrated reduction in simultaneous peak-power draw compared to a baseline (no optimization) scenario, via the throttling/deferral logic.

- A reusable, documented codebase (firmware + Python training scripts + ThingsBoard configuration) suitable for extension to additional bays or real hardware.

# 13. Future Enhancements

- Replace the lightweight on-device model with a more advanced TensorFlow Lite Micro neural network trained on real charging-station datasets.

- Add a public-facing display/app showing live charger availability and estimated wait times to EV owners.

- Integrate real-time electricity tariff APIs for genuinely dynamic, cost-aware scheduling.

- Add OTA firmware updates so the Edge AI model can be retrained and redeployed without physical access to devices.

- Extend the rule chain to send notifications (email/SMS/Telegram) to operators for critical alarms.

- Incorporate renewable energy source availability (solar) into the optimization decision.

# 14. Technology Stack Summary

| Layer | Technology |
| --- | --- |
| Microcontroller | ESP32 (primary) / Arduino Uno + ESP8266 (alternative) |
| Simulation | Wokwi |
| Firmware Language | C/C++ (Arduino framework) |
| Communication | MQTT (PubSubClient), JSON (ArduinoJson) |
| Edge AI | scikit-learn (training) + micromlgen / m2cgen (C export), optional TensorFlow Lite Micro |
| Cloud Platform | ThingsBoard CE (cloud demo or self-hosted Docker) |
| Visualization | ThingsBoard Dashboards & Widgets |
| Automation/Alerts | ThingsBoard Rule Chains |

# 15. References

- ThingsBoard MQTT Device API Documentation — thingsboard.io/docs

- Wokwi Documentation — docs.wokwi.com

- Espressif ESP32 Arduino Core Documentation

- scikit-learn Documentation — scikit-learn.org

- micromlgen and m2cgen project documentation (model-to-C-code conversion)

- PubSubClient and ArduinoJson library documentation